🌱

Smart AgroSense

AI & Sensor-Based Crop Recommendation System for Nepal

Complete Simulink Model Design Guide

MATLAB Function Block Code  |  Subsystem Architecture  |  ML Integration

| 8 Crops | 4 Sensors | 88% Cost Saved | 0-100 Score Range |
| --- | --- | --- | --- |

01  System Overview

Smart AgroSense is a low-cost, sensor-driven agricultural decision support system designed specifically for Nepal's diverse farming conditions. By replacing expensive NPK sensors with AI-based nutrient estimation, the system reduces hardware cost by over 88% while maintaining high recommendation accuracy.

System Goals ✓ Collect real-time soil and environment data using 4 affordable sensors ✓ Filter noisy sensor signals using moving average + IIR low-pass filters ✓ Classify soil conditions (pH, moisture, temperature, humidity) ✓ Use rule-based AI + ML models to recommend the best crop from 8 options ✓ Display farmer-friendly advice in plain language ✓ Demonstrate 88% cost reduction vs traditional NPK-sensor systems

## Crop Coverage

| Rice | Maize | Wheat | Mustard | Potato | Tomato | Lentil | Millet |
| --- | --- | --- | --- | --- | --- | --- | --- |

02  Simulink Model Architecture

The Simulink model is organized into 8 interconnected subsystems. The data flows left-to-right from sensor generation through to the dashboard display.

## Signal Flow Diagram

| SENSOR DATA GENERATION Sine Wave + Band-Limited White Noise → pH ¦ Moisture ¦ Temp ¦ Humidity |
| --- |
| ▼ Raw signals |
| SIGNAL FILTERING Moving Average (10-sample window) + IIR Low-Pass Filter (α=0.2) |
| ▼ Filtered signals (2 parallel paths) |
| SOIL CLASSIFICATION pH class ¦ Moisture class ¦ Temp class ¦ Humidity class | AI CROP RECOMMENDATION Trapezoid scoring × 8 crops → Best crop + Score + Advice |
| ▼ Crop index + Score + Advice code |
| FARMER ADVICE GENERATION Advice Code → Human-readable recommendation string |
| ▼ Display outputs |
| DASHBOARD DISPLAY Display blocks + Scope (raw vs filtered) + Cost Comparison |

03  Subsystem-by-Subsystem Block Design

## 3.1  Sensor Data Generation Subsystem

Four sensor channels are simulated using Sine Wave blocks for the base signal plus Band-Limited White Noise blocks to represent real sensor noise. Each channel uses realistic values for Nepal agriculture.

| Sensor | Range | Mean | Sine Amp | Noise Cov | Freq (rad/s) |
| --- | --- | --- | --- | --- | --- |
| Soil pH | 4.5 – 8.5 | 6.5 | 2.0 | 0.04 | 0.05 |
| Soil Moisture % | 10 – 90 | 50 | 30.0 | 4.0 | 0.03 |
| Temperature °C | 10 – 40 | 25 | 12.0 | 1.0 | 0.02 |
| Air Humidity % | 30 – 95 | 65 | 25.0 | 9.0 | 0.04 |

### Simulink Blocks Required:

- Sources > Sine Wave — set Amplitude, Bias, Frequency, Phase, SampleTime=0.1

- Sources > Band-Limited White Noise — set Noise power (Cov) and Sample time=0.1

- Math Operations > Add — set Inputs to ++ (two inputs, both positive)

- Signal Routing > Outport — one per sensor channel

## 3.2  Signal Filtering Subsystem

Each sensor channel passes through two sequential filters to remove noise and smooth the signal. Scope blocks show the before/after comparison.

Filter Stage 1: Moving Average Block: Discrete > Discrete Transfer Fcn Numerator: ones(1,10)/10 (uniform 10-sample window) Denominator: [1] Sample Time: 0.1 seconds Effect: Reduces high-frequency noise while preserving slow trends

Filter Stage 2: IIR Low-Pass Filter Block: Discrete > Discrete Transfer Fcn Numerator: [0.2] Denominator: [1, -0.8] Transfer function: H(z) = 0.2 / (z - 0.8) ←→ y[n] = 0.8·y[n-1] + 0.2·x[n] Smoothing constant α = 0.2 (lower = smoother, higher = more responsive)

Scope Setup (Raw vs Filtered) Add a Scope block with NumInputPorts = 2 Port 1: Raw sensor signal (from Sensor Data Generation subsystem output) Port 2: Filtered signal (from LPF output) Open Scope Properties > set legend labels to "Raw" and "Filtered"

## 3.3  Soil Classification Subsystem

A MATLAB Function block classifies each filtered sensor reading into categorical levels suitable for Nepal farming. Classes are encoded as integers for signal routing.

| Parameter | Code | Class Label | Range / Condition |
| --- | --- | --- | --- |
| pH | 1 | Strongly Acidic | pH < 5.5 |
| pH | 2 | Slightly Acidic | 5.5 ≤ pH < 6.5 |
| pH | 3 | Neutral | 6.5 ≤ pH ≤ 7.5 |
| pH | 4 | Alkaline | pH > 7.5 |
| Moisture | 1 | Dry | < 25% |
| Moisture | 2 | Moderate | 25% – 60% |
| Moisture | 3 | Wet | > 60% |
| Temperature | 1 | Too Low | < 15°C |
| Temperature | 2 | Suitable | 15°C – 32°C |
| Temperature | 3 | Too High | > 32°C |
| Humidity | 1 | Low | < 45% |
| Humidity | 2 | Moderate | 45% – 70% |
| Humidity | 3 | High | > 70% |

### MATLAB Function Block — Soil Classifier Code:

function [ph_class, moisture_class, temp_class, humidity_class] = ...

soil_classifier(ph_val, moist, temp, humidity)

% pH Classification

if ph_val < 5.5,         ph_class = 1;   % Strongly Acidic

elseif ph_val < 6.5,     ph_class = 2;   % Slightly Acidic

elseif ph_val <= 7.5,    ph_class = 3;   % Neutral

else,                    ph_class = 4;   % Alkaline

end

% Moisture Classification

if moist < 25,           moisture_class = 1;   % Dry

elseif moist <= 60,      moisture_class = 2;   % Moderate

else,                    moisture_class = 3;   % Wet

end

% Temperature Classification (Nepal growing seasons)

if temp < 15,            temp_class = 1;   % Too Low

elseif temp <= 32,       temp_class = 2;   % Suitable

else,                    temp_class = 3;   % Too High

end

% Humidity Classification

if humidity < 45,        humidity_class = 1;   % Low

elseif humidity <= 70,   humidity_class = 2;   % Moderate

else,                    humidity_class = 3;   % High

end

end

04  AI Crop Recommendation Engine

The core of AgroSense is a rule-based fuzzy-logic engine that calculates a suitability score from 0–100 for each of 8 Nepal crops using a trapezoidal membership function on each input parameter.

## Trapezoidal Membership Function

How the Scoring Works For each crop and each parameter, a trapezoidal function assigns a 0–100 score: • Score = 0 if value is below minimum or above maximum • Score = 0→100 linearly from minimum to optimal-low boundary • Score = 100 if value is within the optimal range [opt_lo, opt_hi] • Score = 100→0 linearly from optimal-high to maximum boundary Final score = weighted sum: 30% pH + 25% moisture + 25% temp + 10% humidity + 10% NPK

## Crop Requirement Table

| Crop | pH min | pH opt | pH max | Moist opt | Temp opt (°C) | Hum opt (%) | NPK min | Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rice | 5.0 | 6.0–7.0 | 8.0 | 65–80% | 25–35°C | 70–90% | 60 | Heavy feeder |
| Maize | 5.5 | 6.0–7.0 | 8.0 | 45–65% | 22–32°C | 55–80% | 55 | Medium-high |
| Wheat | 5.5 | 6.0–7.0 | 8.0 | 40–60% | 15–22°C | 50–70% | 50 | Medium |
| Mustard | 5.5 | 6.0–7.0 | 7.5 | 35–55% | 15–25°C | 45–65% | 40 | Light |
| Potato | 4.8 | 5.5–6.5 | 7.0 | 55–70% | 18–24°C | 60–80% | 65 | Heavy feeder |
| Tomato | 5.5 | 6.0–6.8 | 7.5 | 50–65% | 22–28°C | 55–75% | 60 | Medium-high |
| Lentil | 5.8 | 6.5–7.5 | 8.0 | 35–50% | 18–25°C | 45–65% | 35 | Light feeder |
| Millet | 5.0 | 5.5–6.5 | 7.5 | 35–55% | 25–35°C | 45–70% | 30 | Very light |

### Core AI Engine MATLAB Function Block Code:

function [top_score, crop_index, advice_code] = ...

crop_ai_engine(ph_f, moist_f, temp_f, hum_f, npk_est)

% Crop requirement table [8 crops x 17 parameters]

cropTable = [

5.0,6.0,7.0,8.0, 50,65,80,90, 20,25,35,40, 60,70,90,95, 60; % Rice

5.5,6.0,7.0,8.0, 30,45,65,80, 18,22,32,38, 40,55,80,90, 55; % Maize

5.5,6.0,7.0,8.0, 25,40,60,70, 10,15,22,28, 35,50,70,85, 50; % Wheat

5.5,6.0,7.0,7.5, 20,35,55,70, 10,15,25,30, 30,45,65,80, 40; % Mustard

4.8,5.5,6.5,7.0, 40,55,70,80, 14,18,24,28, 45,60,80,90, 65; % Potato

5.5,6.0,6.8,7.5, 35,50,65,75, 18,22,28,35, 40,55,75,85, 60; % Tomato

5.8,6.5,7.5,8.0, 20,35,50,65, 12,18,25,30, 30,45,65,75, 35; % Lentil

5.0,5.5,6.5,7.5, 20,35,55,70, 20,25,35,40, 30,45,70,85, 30; % Millet

];

scores = zeros(1,8);

for c = 1:8

req = cropTable(c,:);

scores(c) = 0.30*trapz(ph_f,    req(1:4))  + ...

0.25*trapz(moist_f, req(5:8))  + ...

0.25*trapz(temp_f,  req(9:12)) + ...

0.10*trapz(hum_f,   req(13:16))+ ...

0.10*npk_s(npk_est, req(17));

end

[top_score, crop_index] = max(scores);

top_score = round(top_score);

% Advice code logic

if ph_f < 5.5,       advice_code = 1;  % Add lime

elseif ph_f > 7.8,   advice_code = 2;  % Add sulfur

elseif moist_f < 25, advice_code = 3;  % Irrigate

elseif moist_f > 75, advice_code = 4;  % Drain

elseif npk_est < 35, advice_code = 5;  % Add fertilizer

else,                advice_code = 6;  % Conditions optimal

end

end

05  AI-Based NPK Estimation (Cost Reduction)

Instead of an expensive NPK sensor ($80–$150), AgroSense estimates nutrient availability from pH and moisture readings using a heuristic model. This is the key innovation enabling 88% cost reduction.

NPK Estimation Logic Biological basis: nutrient availability is pH-dependent (Hoagland model) • N, P, K are most available at pH 6.0–7.0 • Below pH 5.5: P becomes fixed; Al/Mn toxicity reduces uptake • Above pH 7.5: Fe, Mn, Cu, Zn become insoluble • Soil moisture affects mass flow and diffusion of ions to roots Formula: pH_factor = max(0, 1 - ¦pH - 6.5¦ / 2.0) [0 to 1 scale] moist_factor = clamp((moisture - 20) / 50, 0, 1) [0 to 1 scale] NPK_est = 100 × (0.6 × pH_factor + 0.4 × moist_factor)

06  Cost Comparison Subsystem

| Component | Traditional System | AgroSense | Savings |
| --- | --- | --- | --- |
| NPK Sensor | $120 (hardware) | AI estimated | $120 saved |
| pH Sensor | $30 | $10 (low-cost) | $20 saved |
| Moisture Sensor | $25 | $8 | $17 saved |
| Temp+Humidity | $30 | $10 | $20 saved |
| Microcontroller | $50 (Arduino Mega) | $12 (ESP32) | $38 saved |
| Display Module | $40 | $0 (smartphone) | $40 saved |
| PCB + Housing | $55 | $0 (3D print) | $55 saved |
| TOTAL | $350 | $40 | $310 (88.6%) |

### Simulink Cost Block Setup:

- Add Constant block: Value=350 (Traditional system cost USD)

- Add Constant block: Value=40 (AgroSense cost USD)

- Add Math Operations > Add (+-): Computes cost difference = 350 - 40 = 310

- Add Math Operations > Divide: ratio = (350-40)/350 = 0.886

- Add Math Operations > Gain: Gain=100 → output = 88.6% reduction

- Connect outputs to Display blocks labeled "Cost Saved (%)" and "System Cost (USD)"

07  Optional Machine Learning Integration

For higher accuracy, AgroSense includes an optional ML prediction block. A Decision Tree, KNN, and Random Forest are trained on synthetic Nepal crop data and integrated via MATLAB Function blocks.

## Model Training Workflow

- Run train_agrosense_ml_model.m to generate training data and fit models

- 2,000 synthetic samples across 8 crop classes (250 per class)

- Feature vector: [pH, Moisture, Temperature, Humidity]

- Train/test split: 80% / 20%

- Expected Decision Tree accuracy: ~75–85%

- Expected Random Forest accuracy: ~82–90%

- Compact models saved as .mat files for Simulink deployment

## Simulink Integration Steps

- Step 1: Run train_agrosense_ml_model.m and verify .mat files are created

- Step 2: Add MATLAB Function block named "ML_Predict_Block" to your model

- Step 3: Paste code from ml_predict_block_fcn.m into the block editor

- Step 4: Connect 4 input ports: ph_f, moist_f, temp_f, hum_f from Filter subsystem

- Step 5: Connect 2 output ports: crop_pred (index 1-8), confidence (0-100)

- Step 6: Add Display block for confidence score; compare with rule-based output

### MATLAB Function Block — Simplified Decision Tree (no .mat needed):

function [crop_pred, confidence] = ml_predict_simple(ph_f, moist_f, temp_f, hum_f)

%#codegen

if ph_f < 5.8

if temp_f < 20

if moist_f < 40,  crop_pred=4; confidence=72; % Mustard

else,             crop_pred=5; confidence=68; % Potato

end

else

if hum_f > 70,    crop_pred=1; confidence=75; % Rice

else,             crop_pred=8; confidence=65; % Millet

end

end

elseif ph_f < 6.8

if temp_f < 20

if moist_f < 45, crop_pred=3; confidence=78; % Wheat

else,            crop_pred=5; confidence=70; % Potato

end

elseif temp_f < 28

if moist_f > 60, crop_pred=6; confidence=74; % Tomato

else,            crop_pred=2; confidence=76; % Maize

end

else

if hum_f > 75,   crop_pred=1; confidence=80; % Rice

else,            crop_pred=2; confidence=71; % Maize

end

end

else   % pH >= 6.8

if temp_f < 22 || moist_f < 45,  crop_pred=7; confidence=73; % Lentil

elseif moist_f < 65,              crop_pred=3; confidence=72; % Wheat

else,                             crop_pred=1; confidence=65; % Rice

end

end

end

08  Dashboard & Farmer Advice Display

The dashboard translates numeric sensor outputs and crop indices into farmer-readable advice. Display blocks show real-time values; Scope blocks compare raw vs filtered signals.

## Dashboard Block Configuration

| Block Name | Block Type | Input Signal | Display Format |
| --- | --- | --- | --- |
| pH_Display | Sinks > Display | Signal_Filtering/1 | short (2 decimal) |
| Moisture_Display | Sinks > Display | Signal_Filtering/2 | short (1 decimal) |
| Temp_Display | Sinks > Display | Signal_Filtering/3 | short (1 decimal) |
| Humidity_Display | Sinks > Display | Signal_Filtering/4 | short (1 decimal) |
| CropScore_Display | Sinks > Display | AI_Crop_Recommendation/1 | short (0 decimal) |
| CropIndex_Display | Sinks > Display | AI_Crop_Recommendation/2 | short (0 decimal) |
| AdviceCode_Display | Sinks > Display | AI_Crop_Recommendation/3 | short (0 decimal) |
| CostSave_Display | Sinks > Display | Cost_Comparison/1 | short (1 decimal) |
| pH_Scope | Sinks > Scope | Raw + Filtered pH (2 ports) | 2 signals overlay |
| Moisture_Scope | Sinks > Scope | Raw + Filtered Moisture | 2 signals overlay |

### Example Farmer Advice Messages (by Advice Code):

| Code | Farmer Message |
| --- | --- |
| 1 | Soil is strongly acidic (pH < 5.5). Maize is recommended. Add agricultural lime (2-3 ton/ha) to raise soil pH before sowing. |
| 2 | Soil is alkaline (pH > 7.8). Lentil is recommended. Apply sulfur or organic compost to reduce pH for better nutrient availability. |
| 3 | Soil moisture is very low (< 25%). Millet is recommended. Irrigate the field immediately before planting. |
| 4 | Soil moisture is too high (> 75%). Rice is recommended. Improve field drainage to prevent root rot in other crops. |
| 5 | NPK levels are estimated low. Wheat is recommended. Apply balanced NPK fertilizer (10-10-10) at recommended rates before planting. |
| 6 | Soil is slightly acidic and moisture is moderate. Maize is recommended. Conditions are optimal - maintain current farming practices. |

09  Step-by-Step Build Guide

Follow these steps to build the complete SmartAgroSense Simulink model from scratch.

Quick Start — Automated Build 1. Open MATLAB and set working directory to the AgroSense project folder 2. Run: build_agrosense_model.m This creates SmartAgroSense.slx with all subsystems and connections 3. For each MATLAB Function block, paste the code from the companion .m files 4. Run the standalone simulation: run_agrosense_simulation.m This validates all logic with plots (no Simulink license needed) 5. Train ML models: train_agrosense_ml_model.m (optional)

## Manual Simulink Build Steps

- Open MATLAB → Home → New → Simulink Model → Blank Model. Save as SmartAgroSense.slx

- Set Simulation Stop Time to 100, Solver Type to Fixed-step, Fixed Step Size to 0.1

- Add Subsystem block (Simulink Library > Ports & Subsystems). Name it Sensor_Data_Generation

- Inside: Add 4x Sine Wave + 4x Band-Limited White Noise + 4x Add + 4x Outport

- Configure each Sine Wave per the sensor table in Section 3.1

- Add second Subsystem: Signal_Filtering. Inside: Add 4x [Inport > MovAvg > LPF > Outport]

- Add third Subsystem: Soil_Classification. Inside: MATLAB Function block + 4 Inports + 4 Outports

- Open MATLAB Function block, paste soil_classifier_fcn.m code, verify function signature

- Add fourth Subsystem: AI_Crop_Recommendation. Inside: MATLAB Function + 5 Inports + 3 Outports

- Paste crop_ai_engine_fcn.m code into the AI engine MATLAB Function block

- Add NPK_Constant block (Constant=50) at top level → connect to AI Engine input 5

- Add Cost_Comparison Subsystem with two Constants (350, 40) + Add + Divide + Gain blocks

- Add 8x Display blocks + 4x Scope blocks (NumInputPorts=2) at top level

- Wire all subsystems: Sensor→Filter→(Classifier, AI Engine)→Dashboard

- Connect raw + filtered signals to respective Scope blocks for comparison view

- Press Run (Ctrl+T) — observe Display blocks updating and Scopes showing noise removal

10  Companion File Reference

| File | Purpose | How to Use |
| --- | --- | --- |
| build_agrosense_model.m | Auto-builds complete Simulink model programmatically | Run first in MATLAB to generate SmartAgroSense.slx |
| soil_classifier_fcn.m | Soil classification MATLAB Function block code | Paste into Soil_Classifier block editor |
| crop_ai_engine_fcn.m | Rule-based AI recommendation engine block code | Paste into Crop_AI_Engine block editor |
| run_agrosense_simulation.m | Standalone pure-MATLAB simulation & validation | Run to test logic + generate plots (no Simulink needed) |
| train_agrosense_ml_model.m | Trains Decision Tree, Random Forest, KNN models | Run once to create .mat model files |
| ml_predict_block_fcn.m | ML prediction MATLAB Function block code | Paste into ML_Predict_Block in optional ML subsystem |
| SmartAgroSense.slx | Final Simulink model (generated by builder script) | Open in Simulink after running builder script |

Requirements & Compatibility MATLAB version: R2020b or later (R2022a+ recommended for Dashboard blocks) Required toolboxes: Simulink, Statistics and Machine Learning Toolbox (for ML block) Optional: Simulink Dashboard Toolbox (for gauge/indicator displays) Hardware deployment: Compatible with MATLAB Support Package for Arduino/Raspberry Pi Standalone simulation (run_agrosense_simulation.m): Requires only base MATLAB

Smart AgroSense  |  Designed for Nepal Smallholder Farmers  |  Low-Cost AI Agriculture